DataSockets

Basic Use of DataSockets in LabVIEW:

Tips, Tricks, and a Few Things to Watch Out For

David Thomson

Original Code Consulting

www.originalcode.com

Paul Sweat

National Instruments

www.ni.com

Boulder LabVIEW User Group Meeting

July 12, 2001

DataSocket:

A simple way to transmit data over a network

No handshaking – basically a broadcasting protocol

Low-level details are all handled under the hood

Much simpler than using TCP/IP primitives

Can transmit any data type (DSTP), as well as files, ftp, opc, logos

Connections made with standard URLs

dstp://servername.com/datatagname

ftp://ftp.natinst.com/datasocket/ping.wav

file:\\machine\mydata\ping.wav

The rest of this presentation deals with transmitting data using DSTP (DataSocket Transfer Protocol).  To use this protocol, the DataSocket Server must be running on a machine accessible on your network.  Before running the DataSocket Server, first configure it with the DataSocket Server Manager.

DataSocket Server Manager

Configure server settings, access rights, and predefined tags

Default is to allow all readers, only local writers and creators

How do you specify a range of IPs for access rights?

(I have not gotten wildcards (*) to work.  Paul Sweat says this should

work.)

DataSockets can have static (predefined) and dynamic items

Use the DS Server Manager to create the predefined items

What are the advantages of predefined tags?

They are limited to a few data types.

You can specify whether multiple writers are allowed.

Can this be specified with dynamic items?

They allow you to specify permissions for each tag, rather than just for

overall read, write, or create access.

DataSocket Server

Must run on one machine – local or remote

DataSockets are cross-platform, but the server is Windows only

(based on ActiveX)

Can start the server programmatically on same machine

See DataSocket Launch.vi example.

Some documentation is unclear on this.  Does anyone know how to start

the Server programmatically on a remote machine?

Jim Filla suggested using VI Server on the local machine to start a

VI on the remote machine which then starts the DataSocket Server

locally.

DataSockets in LabVIEW

DS under 5.1 was more difficult and supported limited data types

LV 6i also added direct connection to a control or indicator

See DS Publish Control.vi and DS Read Control.vi examples

Right-click on the control, select Data Operations/Data Socket Connection

Red or Green light indicates the connection status

If both VI’s are set to Publish AND Subscribe, then the first one to run gets

to Publish and Subscribe, the second can only Subscribe and will get a red

indicator light, even though it Subscribes correctly.  However, if both VIs

are on the same computer, then both can Publish and Subscribe.

There is no apparent control over the rate at which data is transferred

The consensus is that a control publishes data when the value changes, and

a subscribing control or indicator will get new data from the DS Server

whenever the data changes.

A bug in 6.0 – doesn’t support some data types (enum controls, probably)

Upgrade to 6.0.2 fixes this

Unfortunately, there is no error code reported; the data just doesn’t get

transmitted.  Default values appear in the indicator.

DS under 6i has simplified VI’s

Check out the DataSocket palette under the Communications functions.

Additional DS functions are in vi.lib\platform\dataskt.llb

These are compatibility VIs for LV5

See the DS Writer and DS Reader examples

Easy programming

Documentation is too sparse

NI has acknowledged – documentation will be improved

Error codes and error handling are sub-optimal

Sometimes –2gajillion (ActiveX errors?) – these aren’t recognized

by LV’s built-in error handling

Sometimes don’t report an error when it should

For example, use DS Reader to read an item before DS

Writer has created it.  DS Reader creates the tag and

returns a default value with no error.  Shouldn’t this

generate an error or warning?

In the above example, the Wait For New Value input is set to

False.  If it is set to True, DS will time-out and the time-

out output will be set, but there is no error reported.

Why is there no DataSocket Browser?

An application associated with the DS Server that could display all current

tags, when they were created, last written, by whom, last read, by

whom, and ideally, their data values.  Simple data types could

easily be displayed.  More complex types may need to be displayed

as variants.  This could be a valuable debugging tool.  When data

isn’t transmitted, was it a failure of the writer or of the reader?

There is a VI called DataSocket Select URL that appears that it might allow

you to see the current tags in the DS Server, but this doesn’t seem to work.

There is a bug in DataSocket/LabVIEW that can be demonstrated as follows.  NI  has acknowledged, will fix it, no date for a bug fix is available.  Similar problems have been discussed on Info-LabVIEW.

Close DataSocket Server, and Exit LabVIEW.

This example requires two networked computers.

Make sure DataSocket server is configured to allow Create and Write access as well as Read for the second computer.

Open DS Writer w Presend.vi on the computer with the DataSocket Server.

Run it.

Enter a non-zero value in the top Number control, then press Write ONCE only.

Note that this program launches the DataSocket Server itself.  However, the problem occurs even if the DataSocket Server is already running when the program is launched.

This program first launches the DS Server, then writes the default values of Presend1 and Presend2.  It then enters a loop.  In the loop, it Writes "Number" whenever the Write button is pressed.

On the second computer, start LabVIEW fresh and open DS Reader w Presend.vi.

Enter the IP address of the first computer.

Start the DS Reader w Presend.vi.

Press the Read button.

*** The value of Number is not read, and no error message is returned!

This program first reads Presend1 and Presend2, then enters a loop.  The Number item is read in the loop whenever the Read button is pressed.  Note that both Presend1 and Presend2 received the correct values, but Number did not.

On DS Writer w Presend.vi, again press Write.

On DS Reader w Presend.vi, again press Read.

The correct value will now be read.

To duplicate the problem, both programs must be stopped and the DataSockets Server must be shut down.  Then the exact order above must be followed.  Doing things differently may cause the problem not to appear.  For example, just exiting LV on the second computer and starting the Read w Presend.vi again will allow the Number value to be read correctly.  Generally, if the Writer w Presend.vi is stopped and restarted, the problem will appear again.  (Note that if the DataSockets Server is stopped, then the Reader w Presend.vi will have to be stopped and started again as it reports that its connection was lost.  (Write w Presend.vi can be stopped without stopping the DataSockets Server by using the Stop Icon on the toolbar instead of the Stop button.))

*** Note that this problem cannot be duplicated when both VI's are run on the same machine.

*** This example is a simplified version of a program I wrote that has a similar problem.  However, in my other program, one of the two "Present" items fails to be transmitted correctly, whereas the items inside the loop appear to work OK.

How is the Write or Read inside the loop different from the Write or Read outside the loop?

Is the Write failing or the Read?  (Here is where a DataSocket Browser would help.)

***A partial fix is to write an item twice before attempting to read it.  This appears to have fixed most but not all of the problems I have run into.

Another apparent bug can be demonstrated with the program datasocket%20test%20bug.vi  (This bug was posted to the Info-LabVIEW e-mail list and is reproduced here with permission of the original programmer.)  Run the program with the Loop? control set to False.  The value of both items are transmitted correctly.  Run the program with Loop? set TRUE, and the value of the first item comes across as 0 rather than 4.  The only difference is that in the first case, two parallel DS Writes are used, whereas in the second case DS Write is in a loop that indexes the URLs and data.